Home Cybersecurity Lab

Project Build & Configuration Writeup

Author: Alex  |  Date: February 2026  |  Version: 1.0

# 1. Project Overview

This project documents the design, build, and configuration of a home cybersecurity lab environment. The lab is built entirely using VirtualBox on a Windows host machine and consists of three virtual machines: a pfSense firewall/router, a Security Onion SIEM/NSM platform, and an Ubuntu endpoint. The goal is to simulate a real enterprise-like network where all traffic is routed through a firewall and monitored by a dedicated security platform.

# 2. Lab Architecture

## 2.1 Network Topology

The lab is divided into two network segments:

- Home Network (192.168.0.0/24): The real home network provided by the ISP router. Security Onion and pfSense WAN live here.

- Lab Network / labnet (10.1.1.0/24): An isolated internal VirtualBox network. pfSense LAN and Ubuntu VM live here. All lab traffic is routed through pfSense.

## 2.2 Traffic Flow

Ubuntu VM → pfSense (10.1.1.1) → Security Onion (192.168.0.118) → Alerts/Dashboards

- Ubuntu generates endpoint and network traffic

- pfSense routes all lab traffic, enforces firewall rules, and forwards logs via syslog

- Security Onion ingests logs from pfSense (syslog UDP 514) and endpoint agents (Elastic Agent port 8220)

## 2.3 Virtual Machine Specifications

- pfSense: 1 CPU, 2GB RAM, 2 NICs (Bridged WAN + Internal labnet LAN)

- Security Onion 2.4.201: 4 CPU, 16GB RAM, 3 NICs (Bridged management + Internal labnet monitor + Internal labnet agent)

- Ubuntu 22.04: 2 CPU, 4GB RAM, 1 NIC (Internal labnet)

# 3. pfSense Setup & Configuration

## 3.1 VirtualBox Network Adapters

- Adapter 1 (WAN): Bridged Adapter — gets DHCP IP from real home router (192.168.0.247)

- Adapter 2 (LAN): Internal Network named labnet — pfSense assigns itself 10.1.1.1/24

## 3.2 Initial Configuration

- WAN interface: DHCP from real router, received 192.168.0.247

- LAN interface: Static 10.1.1.1/24, provides DHCP to lab VMs

- Web GUI accessible at https://10.1.1.1 from lab VMs

- Default admin password changed on first login

## 3.3 Firewall Rules

LAN Rules:

- Anti-Lockout Rule (auto-generated): allows GUI access on ports 443/80

- Default allow LAN to any: permits all outbound lab traffic

- LAN subnets → 192.168.0.118:8220 (Elastic Agent enrollment)

- LAN subnets → 192.168.0.118:1514-1515 (Wazuh/agent communication)

WAN Rules:

- Block private networks: DISABLED (was blocking lab-to-SIEM traffic)

- Block bogon networks: DISABLED

- WAN address → 192.168.0.118:514 (syslog forwarding to Security Onion)

## 3.4 Syslog Forwarding to Security Onion

Configured under Status > System Logs > Settings > Remote Logging:

- Remote log server: 192.168.0.118:514

- Events forwarded: Firewall Events, DHCP Events, General System Events

- Result: 20,000+ logs ingested into Security Onion within minutes of configuration

# 4. Security Onion Setup & Configuration

## 4.1 Installation

- Version: Security Onion 2.4.201

- Installation type: Evaluation (single-node)

- Stack: Standard (not airgapped)

- Hostname: siem

## 4.2 Network Adapter Configuration

Final working adapter configuration (order matters for setup wizard):

- Adapter 1: Internal Network (labnet) — monitor interface, passively captures all lab traffic

- Adapter 2: Bridged Adapter — management interface, IP: 192.168.0.118 (home network)

- Adapter 3: Internal Network (labnet) — added post-install for direct agent connectivity from lab VMs, IP: 10.1.1.13

## 4.3 Management Interface Settings

- IP Address: 192.168.0.118/24

- Gateway: 192.168.0.1 (home router)

- DNS: 8.8.8.8

- Search Domain: home.lab

- Web UI access restricted to: 192.168.0.113 (Windows host)

## 4.4 Security Onion Firewall Configuration

Added lab network to the analyst hostgroup under Administration > Configuration > firewall > hostgroups > analyst:

- 10.1.1.0/24 added to allow lab VM connections

- iptables rule manually added: 192.168.0.247 (pfSense WAN) allowed inbound

- Applied with: sudo so-firewall apply

# 5. Ubuntu VM Setup

## 5.1 Configuration

- OS: Ubuntu 22.04 LTS

- Network: Internal Network (labnet), receives DHCP from pfSense (10.1.1.x)

- Default gateway: 10.1.1.1 (pfSense) — all traffic routes through firewall

## 5.2 Elastic Agent Installation

Security Onion 2.4 uses Elastic Agent instead of Wazuh for endpoint monitoring. Downloaded from Security Onion Downloads section and installed:

- Installer downloaded directly on Ubuntu from Security Onion Downloads page

- Fleet URL: https://10.1.1.13:8220 (Security Onion labnet IP)

- Enrollment token pre-configured in installer from Security Onion Downloads

## 5.3 Routing Configuration

Static route added to ensure lab traffic goes through pfSense:

sudo ip route add 192.168.0.118 via 10.1.1.1

# 6. Errors Encountered & Resolutions

Documenting errors and their fixes is valuable for troubleshooting and demonstrates real hands-on experience.

## 6.1 VirtualBox UUID Conflict on Disk Creation

Error:

Cannot register the hard disk 'Ubuntu.vdi' because a hard disk with UUID {b10419bd-cb5b-4e53-83e9-1b7c79d965ad} already exists.

Cause: VirtualBox had a stale registry entry from a previous failed VM creation attempt.

Fix:

VBoxManage closemedium disk {b10419bd-cb5b-4e53-83e9-1b7c79d965ad} --delete

## 6.2 UUID Mismatch on Boot

Error:

UUID of medium does not match value stored in VirtualBox.xml

Cause: VDI file UUID did not match the UUID recorded in VirtualBox's media registry after a failed install.

Fix:

VBoxManage internalcommands sethduuid "C:\Users\Alex\VirtualBox VMs\Ubuntu\Ubuntu.vdi" {UUID}

## 6.3 Security Onion Setup Timeout — sigs.securityonion.net

Error:

resolve host sigs.securityonion.net — timeout, will retry in 60 seconds

Cause: Security Onion's management interface was on a Host-Only network with no internet access. pfSense was also not running to provide routing.

Fix: Started pfSense VM first (provides internet routing for labnet), changed Security Onion management adapter to Bridged so it has direct internet access for the initial install.

## 6.4 Security Onion on Wrong Internal Network

Error: Ubuntu and Security Onion could not communicate despite both being on internal networks.

Cause: Security Onion's monitor adapter was set to an internal network named inet instead of labnet. The two VMs were on completely different isolated networks.

Fix: Changed Security Onion's monitor adapter to Internal Network named labnet to match pfSense LAN and Ubuntu.

## 6.5 pfSense Blocking Private Network Traffic

Error: Traffic from Ubuntu (10.1.1.x) could not reach Security Onion (192.168.0.118) despite correct routing.

Cause: pfSense had Block private networks and Block bogon networks rules enabled on the WAN interface, which blocked RFC 1918 addresses (including 10.1.1.0/24) from being forwarded.

Fix: Disabled both Block private networks and Block bogon networks rules in Firewall > Rules > WAN.

## 6.6 Elastic Agent Cannot Reach Fleet Server

Error:

FleetHostConnectivityCheck=Failed FleetHostURL=https://192.168.0.118:8220

Cause: Security Onion's Fleet server (port 8220) was only bound to the management interface (192.168.0.118), not accessible from the lab network. A third adapter on labnet was needed.

Fix: Added Adapter 3 to Security Onion VM as Internal Network (labnet). Security Onion received IP 10.1.1.13, which Ubuntu can reach directly without NAT complications.

# 7. Verification & Testing

## 7.1 pfSense Verification

- WAN IP confirmed under Status > Interfaces: 192.168.0.247

- Lab VMs receiving DHCP from pfSense in 10.1.1.0/24 range

- Internet access confirmed from lab VMs via ping 8.8.8.8

## 7.2 Security Onion Verification

- Web UI accessible at https://192.168.0.118 from Windows host

- 20,000+ pfSense logs ingested within minutes of syslog configuration

- Elasticsearch Status: OK, Connection Status: OK

- Fleet server accessible from labnet at https://10.1.1.13:8220

## 7.3 Useful Verification Commands

Ubuntu — confirm routing through pfSense:

ip route

traceroute 192.168.0.118

ping 10.1.1.1

Security Onion — check services and firewall:

sudo so-status

sudo iptables -L INPUT | head -20

sudo docker ps

# 8. Next Steps

- Complete Elastic Agent enrollment on Ubuntu and verify endpoint logs appear in Security Onion dashboards

- Install Suricata on pfSense for IDS/IPS alerts fed into Security Onion

- Write custom detection rules in Security Onion for failed login brute force detection

- Add Windows VM to lab and enroll as additional monitored endpoint

- Simulate attacks (nmap, Metasploit) from a Kali VM and observe alerts in Security Onion

- Document and push all writeups to GitHub repository for portfolio

# 9. Lessons Learned

- VirtualBox internal network names are case-sensitive and must match exactly across all VMs — a mismatch creates isolated networks that cannot communicate

- pfSense Block private networks rule on WAN will silently drop RFC 1918 traffic — always disable in a lab environment

- Security Onion must have internet access during initial setup to pull signatures — pfSense must be running first or use a Bridged adapter

- NAT in pfSense changes the source IP of packets — Security Onion sees pfSense WAN IP (192.168.0.247), not the original lab VM IP

- Security Onion 2.4 uses Elastic Agent not Wazuh — always verify agent type matches your Security Onion version before installing

- Taking VM snapshots before major changes saves significant time when troubleshooting